End to End Small Office Network Project Documentation

1. Project Overview

This project simulates a small office network designed using Cisco networking concepts aligned with CCNA topics.

The goal is to design, configure, secure, and troubleshoot a functional office network that includes:

- VLAN segmentation

- Inter VLAN routing

- DHCP and DNS services

- OSPF dynamic routing

- WAN branch connectivity

- Access Control Lists

- Wireless integration

- Basic monitoring readiness

This lab reflects a real SME network environment.

2. Network Objectives

- Segment departments using VLANs

- Provide automatic IP addressing

- Enable communication between departments

- Connect branch office via WAN

- Secure Guest access

- Implement basic redundancy awareness

- Prepare environment for monitoring systems

3. Network Topology

Devices Used:

- 2 Cisco Layer 2 switches

- 2 Cisco routers

- 1 Server (DNS)

- 1 Wireless router / AP

- 6 Wired PCs

- 2 Wireless PCs

Logical Design:

Head Office:

- VLAN 10 – Admin

- VLAN 20 – Staff

- VLAN 30 – Guest

Branch Office:

- Single LAN subnet

WAN:

- Serial link between HQ and Branch

4. IP Addressing Plan

Head Office VLANs:

VLAN 10
192.168.10.0/24
Gateway: 192.168.10.1

VLAN 20
192.168.20.0/24
Gateway: 192.168.20.1

VLAN 30
192.168.30.0/24
Gateway: 192.168.30.1

Branch Office:
192.168.40.0/24
Gateway: 192.168.40.1

WAN Link:
10.0.0.0/30

5. VLAN Configuration

Created VLANs:

- VLAN 10 Admin

- VLAN 20 Staff

- VLAN 30 Guest

Access ports assigned accordingly.
Trunk configured between switches and router.

Purpose:

- Reduce broadcast domains

- Improve security

- Separate departments

Verification Commands:

- show vlan brief

- show interfaces trunk

6. Inter VLAN Routing

Router on a stick configuration implemented.

Subinterfaces created:

- G0/0.10

- G0/0.20

- G0/0.30

Encapsulation dot1q configured per VLAN.

Purpose:

- Allow communication between VLANs

- Maintain segmentation while enabling controlled access

Verification:

- show ip route

- ping across VLANs

7. DHCP Configuration

Router configured as DHCP server.

Separate pools created for each VLAN.

Excluded default gateway addresses.

Purpose:

- Automate IP address assignment

- Reduce manual configuration errors

Verification:

- ipconfig on PCs

- show ip dhcp binding

8. DNS Configuration

Server configured as DNS server.

Created A records for:

- internal web server

- branch server

Purpose:

- Enable hostname resolution

- Simulate enterprise name services

Verification:

- ping hostname

- nslookup

9. WAN Connectivity

Serial link configured between HQ and Branch router.

IP addressing assigned using /30 network.

Initially configured static routing.
Later replaced with OSPF area 0.

Purpose:

- Connect remote office

- Learn static vs dynamic routing

Verification:

- show ip route

- show ip ospf neighbor

- End to end ping

10. OSPF Configuration

Configured OSPF area 0 on both routers.

Network statements included all relevant subnets.

Purpose:

- Dynamic route exchange

- Automatic failover logic

- Scalability for future growth

Verification:

- show ip ospf neighbor

- show ip route ospf

11. Access Control Lists

Standard ACL:

- Block Guest VLAN from accessing Admin VLAN

Extended ACL:

- Allow only HTTP and DNS from Guest VLAN

- Deny other traffic

Purpose:

- Enforce network security policy

- Protect sensitive departments

Verification:

- Ping tests

- HTTP simulation tests

12. Wireless Integration

Wireless router added.

Configured:

- Secure SSID

- WPA2 encryption

- Guest VLAN mapping

Purpose:

- Provide mobility

- Maintain segmentation

- Secure wireless access

Verification:

- Wireless client connectivity

- VLAN isolation tests

13. Security Considerations

- Disabled unused switch ports

- Applied basic port security

- Secured console and VTY lines

- Configured enable secret

- Used ACLs to restrict inter VLAN access

Purpose:

- Reduce attack surface

- Protect management plane

14. Testing and Validation

Test Scenarios Performed:

- VLAN isolation test

- Inter VLAN communication test

- DHCP failure simulation

- OSPF neighbor failure test

- WAN link shutdown test

- ACL misconfiguration test

- Wireless authentication test

Each failure was diagnosed using:

- ping

- traceroute

- show ip route

- show ip interface brief

- show vlan brief

15. Troubleshooting Summary

Common Issues Encountered:

- Incorrect VLAN assignment

- Trunk not allowing VLAN

- Missing default gateway

- Incorrect OSPF network statement

- ACL applied in wrong direction

Resolution Approach:

- Identify layer of failure

- Verify physical connectivity

- Check IP configuration

- Verify routing table

- Check ACL order and direction

16. Lessons Learned

- Proper IP planning prevents routing confusion

- VLAN segmentation improves security and performance

- OSPF simplifies multi router management

- ACL placement matters significantly

- Testing failure scenarios improves troubleshooting confidence

17. Skills Demonstrated

- Layer 2 switching

- VLAN segmentation

- Inter VLAN routing

- DHCP and DNS services

- Static and dynamic routing

- WAN configuration

- OSPF configuration

- ACL security implementation

- Wireless configuration

- Network troubleshooting methodology

18. Conclusion

This project demonstrates the ability to design, implement, secure, and troubleshoot a small enterprise network environment using CCNA level technologies.

The lab simulates real world scenarios encountered in SME and branch office deployments.

Author

Dustin Rosario
IT Service Desk | Networking Enthusiast | CCNA Learner